# Smart Plant Watering Project
Technical Build Specification

Purpose: hardware handoff document for coding and final integration

System summary: A 12V power supply feeds the pump directly and also feeds a 5V buck converter. The buck converter powers the ESP32. The ESP32 reads a capacitive soil moisture sensor, drives a pump through a Gravity MOSFET module, and controls two status LEDs. All grounds are common.

# Bill of Materials

| Item | Part | Qty | Notes |
| --- | --- | --- | --- |
| Main controller | ESP32 Development Board – DEVKIT V1 / ESP32-DevKitC | 1 | Powered from 5V buck output; GPIO assignment fixed below. |
| Soil sensor | Gravity Analog Soil Moisture Sensor – Corrosion Resistant | 1 | Pins are A, +, −. Run from ESP32 3.3V. |
| Pump driver | Gravity MOSFET Power Controller | 1 | Control pins use ESP32 3.3V logic. |
| Pump | Peristaltic Liquid Pump 12V DC – Tube 3x5mm, 80 ml/min | 1 | Pump stays on 12V side only. |
| Main power | Power Supply 12V 4A – Jack 5.5/2.1 | 1 | Single external power source for whole system. |
| Buck converter | DC-DC Converter Step-Down 5V 1A | 1 | Pins are IN+, OUT+, GND. GND is shared input/output negative. |
| DC input | DC Power Jack 5.5 x 2.1mm Barrel | 1 | For clean panel or enclosure power entry. |
| Protection diode | 1N4007 rectifier diode | 1 used, extras on hand | Flyback diode across pump terminals; stripe to +12V. |
| Tubing | Silicone Tube Transparent 3x5mm | As needed | Matches pump tubing size. |
| Status LED 1 | 5mm Red LED + 200Ω resistor | 1 | Assigned to GPIO19. |
| Status LED 2 | 5mm Green LED + 200Ω resistor | 1 | Assigned to GPIO18. |
| Wiring | Hookup wire / jumper wires | As needed | User already has wires. |
| Mechanical | 3D printed enclosure / brackets / tube holder / reservoir mount | As needed | To be designed around final layout. |

# Power Architecture

## Single-plug approach:

12V adapter positive is the shared +12V rail.

12V adapter ground is the shared GND rail.

The pump uses the 12V rail directly through the MOSFET switching path.

The buck converter reduces 12V to 5V for the ESP32 only.

All grounds are common: power supply ground, buck ground, ESP32 ground, MOSFET ground, sensor ground, and LED ground.

# GPIO and Signal Assignment

| Function | ESP32 pin | Notes |
| --- | --- | --- |
| Red LED | GPIO19 | Status LED; use series 200Ω resistor. |
| Green LED | GPIO18 | Status LED; use series 200Ω resistor. |
| MOSFET control signal | GPIO26 | Pump on/off control signal to MOSFET SIG pin. |
| Soil moisture analog input | GPIO34 | Sensor A pin to analog-capable input. |
| Sensor power | 3.3V | Sensor + pin goes to ESP32 3.3V. |
| MOSFET logic power | 3.3V | MOSFET control-side + pin goes to ESP32 3.3V. |

# Exact Connectivity

## 1. Main 12V rail

+12V rail connects to Pump +, MOSFET VIN+, and Buck IN+.

GND rail connects to MOSFET GND, Buck GND, and ESP32 GND.

Because the buck board only has IN+, OUT+, and GND, that single GND pin is used as both input negative and output negative.

## 2. Buck converter to ESP32

Buck OUT+ connects to ESP32 5V/VIN pin.

Buck GND connects to ESP32 GND.

Do not connect 12V directly to the ESP32.

## 3. Pump and MOSFET power path

Pump + connects to the shared +12V rail.

Pump − connects to the MOSFET load/output negative terminal (label varies by board, typically OUT− or LOAD−).

MOSFET VIN+ connects to the shared +12V rail.

MOSFET GND connects to the shared GND rail.

Resulting switched power path is: +12V rail → Pump → MOSFET → GND rail.

## 4. Flyback diode across pump

1N4007 diode is placed directly across the pump terminals.

Diode stripe (cathode) goes to Pump + / +12V side.

Diode non-striped side goes to Pump − / MOSFET side.

## 5. MOSFET control-side pins

MOSFET SIG connects to ESP32 GPIO26.

MOSFET + (logic/control VCC) connects to ESP32 3.3V.

MOSFET GND connects to shared GND.

## 6. Soil moisture sensor pins

Sensor A connects to ESP32 GPIO34.

Sensor + connects to ESP32 3.3V.

Sensor − connects to shared GND.

Sensor should run from 3.3V, not 5V.

## 7. LEDs

Red LED path: ESP32 GPIO19 → 200Ω resistor → LED long leg (+); LED short leg (−) → shared GND.

Green LED path: ESP32 GPIO18 → 200Ω resistor → LED long leg (+); LED short leg (−) → shared GND.

LED grounds may return to any common ground point, including MOSFET ground.

# Recommended Software Contract for Coding Agent

Read analog moisture from GPIO34 and expose raw value for calibration.

Drive GPIO26 high/low to control pump through MOSFET.

Use GPIO19 red LED to indicate watering state or error state.

Use GPIO18 green LED to indicate normal idle/healthy state.

Keep pump runtime conservative in first firmware tests, for example 1 to 3 seconds max per cycle.

Add a cooldown delay after watering before reading moisture again, so the soil has time to absorb water.

Treat moisture thresholds as calibration values, not hardcoded universal constants.

# Physical Build Notes

Use one compact +12V distribution point and one compact GND distribution point to avoid jumper-wire clutter.

Keep pump and tubing physically separated from the ESP32 and exposed electronics where practical.

Keep wires from buck converter to ESP32 short.

Route tubing as reservoir → pump inlet → pump outlet → plant.

Use a 3D printed tube holder or spike at the plant side to keep the outlet in place.

Water reservoir is external to this spec and can be a printed tank, bottle, or jar with feed tube and vent.

# Sanity Checks Before Power-Up

Verify no 12V line goes directly to ESP32 pins.

Verify buck OUT+ goes only to ESP32 5V/VIN.

Verify pump + is on +12V and pump − is on MOSFET output, not directly to GND.

Verify diode stripe faces the +12V side of the pump.

Verify all grounds are connected together.

Verify LEDs each have a 200Ω resistor in series.

Verify sensor power is 3.3V.